| Jetbot Part List |  |  |  |  |
| --- | --- | --- | --- | --- |
| Order | Parts | serial number | Quantity | Remark |
| 1 | Main bracket | Chassis | 1 |  |
| 2 | Camera IMX219 Pi Camera V2 | Camera | 1 |  |
| 3 | Intel Dual Band Wireless-Ac 8265 W/Bt | Wi-fi | 1 |  |
| 4 | Dual frequency antenna | Antennas | 2 |  |
| 5 | TT motor | TT | 2 |  |
| 6 | Wheel | Wheel | 2 |  |
| 7 | Motor Driver Board PCA9685+TB6612 | P | 1 |  |
| 8 | Universal wheel | Universal wheel | 1 |  |
| 9 | 10000mAh Power Bank | Power bank | 1 |  |
| 10 | Usb Cable | U | 1 |  |
| 11 | PiOLED display 128x32 OLED | OLED | 1 |  |
| 12 | OLED Board | Pinboard | 1 |  |
| 13 | TF卡 64G | TF | 1 |  |
| Mounting accessories |  |  |  |  |
| 13 | M2*5 | Screw-A | 12 | To fix motherboard, motor board, camera |
| 14 | M3*5 | Screw-B | 2 | To fix universal wheel |
| 15 | M3*30 | Screw-C | 4 | To fix motor |
| 16 | M3 | Nut | 4 |  |
| 17 | M2*5+5。 | Screw-D | 4 | To fix camera |
| 18 | M2*5+5。 | Screw-D | 4 | To fix motherboard |
| Cables |  |  |  |  |
| 19 | Motor Cable |  | 3 | 3 two-color lines |
| 20 | Jetson Nano to driver board cable |  | 8 | 8 double female head lines |

Mounting

| Pinboard | OLED |
| --- | --- |
| Camera | Antennas |
| Power bank | 完成 |

Hardware Connecting

PCA9685+TB6612

Nano Kit

[J41] 40-pin expansion header includes:

• Power pins.

Two 3.3V power pins and two 5V power pins. These are not switchable; power is always available when the developer kit is connected to power.

Two 5V pins can be used to power the developer kit at 3A each.

• Interface signal pins.

All signals use 3.3V levels.

By default, all interface signal pins are configured as GPIOs, except pins 3 and 5 and pins 27 and 28, which are I2C SDA and SCL, and pins 8 and 10, which are UART TX and RX. L4T provides a Python library, Jetson.GPIO, for controlling GPIOs. The library has the same API as RPi.GPIO. See /opt/nvidia/jetson-gpio/doc/README.txt on your Jetson system for details.